Harold’s AC Circuits

Cheat Sheet

11 October 2025

Circuit Laws

| Circuit Law | Formula |  |
| --- | --- | --- |
| Kirchhoff’s Current Law (KCL) | i=1nIi=0 Iin1+Iin2=Iout1+Iout2+Iout3 |  |
|  | The total current flowing into a node or junction must equal the total current flowing out of the node or junction. (conservation of charge) |  |
| Kirchhoff’s Voltage Law (KVL) | i=1nVi=0 V1=V2+V3+V4+V5+V6 |  |
|  | The sum of all voltages around a circuit loop is equal to zero. (conservation of energy) |  |
| Ohm’s Law | V=IR (DC circuit) V=IZEq (AC circuit) |  |
| Power | P=VI P=I2R P=V2R P=Wt | An across (V) times a through (I) variable. Watt (W)=Volt (V)Ampere (A) |
| Electrical Energy | E=QV E=VIt P=I2Rt E=Pt | Joule J=Watt Wsecond s |

Components in Series/Parallel

| Element | Resistor | Capacitor | Inductor |
| --- | --- | --- | --- |
| Component |  |  |  |
| Symbol |  |  |  |
| Denoted by | R | C | L |
| Units | Ω (Ohm) | F (Farad) | H (Henry) |
| Equation | R=VRI | C=QVC | L=VLdidt |
| Series |  |  |  |
|  | RT=R1+R2+R3 | 1CT=1C1+1C2+1C3 | LT=L1+L2+L3 |
| Parallel |  |  |  |
|  | 1RT=1R1+1R2+1R3 | CT=C1+C2+C3 | 1LT=1L1+1L2+1L3 |

Impedance

| Component | Units | Impedance (Z) | Phasor Notation | Complex Notation |
| --- | --- | --- | --- | --- |
| Resistance (R) | Ω (Ohm) | ZR=R | R ∠ 0° | R+0j |
| Inductance (L) | mH (Henry) | ZL=jωL | ZL ∠ 90° | 0+¦ZL¦j |
| Capacitance (C) | µF (Farad) | ZC=1jωC=-j1ωC | ZC ∠-90° | 0-¦ZC¦j |
| Impedance (Z) | Ω (Ohm) | Zeq=ZR+ZL+ZC | Zeq ∠ θ° | ZRe+ZImj |
| Alternating Voltage (V) | V (Volts) | V=IZeq | Veq ∠ θ° | VRe+VImj |
| Alternating Current (I) | A (Ampere) | Ieq=VZeq | Ieq ∠ θ° | IRe+IImj |
| NOTE: In circuits, j is used to denote -1 instead of i, so as not to confuse it with current. |  |  |  |  |

Phasor Math

| Circuit Law | Formula | TI-84 Calculator |
| --- | --- | --- |
| Phasor (Polar) | ¦Z¦ ∠ θ°=Zeθj 80 ∠-30°=80e-30j | Example: 80 ∠-30° [MODE] RADIAN [MODE] re^(θi) [2nd] [QUIT] [8] [0] [2nd] [ex] [-] [3] [0] [2nd] [π] [÷] [180] [2nd] [i] [ENTER] |
| Complex (Rectangular) | ZRe+ZImj | Example: 2+3i [MODE] DEGREE [MODE] a+bi [2nd] [QUIT] [2]+[3][2nd] [i] [ENTER] |
| Addition | Z1 ∠ θ1° + Z2 ∠ θ2° - Convert from polar to rectangular form - Add real to real and imaginary to imaginary - Convert back from rectangular to polar form |  |
| Division | Z1 ∠ θ1°Z2 ∠ θ2°=Z1Z2∠ θ1°-θ2° |  |
| Rectangular → Polar | Z=ZRe+ZImj→Z ∠ θ Z=ZRe2+ZIm2 θ=tan-1 ZImZRe | Example: 2+3i [MODE] DEGREE [MODE] re^(θi) [2nd] [QUIT] [2] + [3] [2nd] [i] [ENTER] [ANS] = 3.61e56.31i [2] + [3] [2nd] [i] [ENTER] [MATH][CPX][ ▶Polar] [ENTER] |
| Polar → Rectangular | Z=Z ∠ θ→ZRe+ZImj ZRe=Zcosθ ZIm=Z sin(θ) | Example: 80 ∠-30° [MODE] RADIAN [MODE] a+bi [2nd] [QUIT] 8] [0] [2nd] [ex] [-] [3] [0] [2nd] [π] [÷] [180] [2nd] [i] [ENTER] [ANS] = 69.28 – 40i 8] [0] [2nd] [ex] [-] [3] [0] [2nd] [π] [÷] [180] [2nd] [i] [ENTER] [MATH][CPX][ ▶Rect] [ENTER] |

Resonance

| Term | Formula | TI-84 Calculator |
| --- | --- | --- |
| Frequency (Hz) | ω=2πf ω°sec=ωπ180° radsec | [MODE] DEGREE [MODE] RADIAN |
| Resonance Frequency | ZL+ZC=0 ω0=1LCrads f0=12πLCHz |  |

AC Circuit Example #1: Series RLC

V=ZeqI

V=ZR+ZL+ZCI

V=R+jωL+1jωCI=R+jωL-j1ωCI

AC Circuit Example #2: Parallel RLC

V=ZeqI

V=11ZR+1ZL+1ZCI=11R+1jωL+jωCI

AC Circuit Example #3: Series (RL) and Parallel (RC)

- Determine the impedance from series and parallel components using complex notation.

Zeq=ZR1+ZL+11ZR2+1ZC

Zeq=R1+jωL+11R2+11jωC

- Substitute values.

Zeq=4+(0.25)10-3ωj+112-(0.1)(10-6)ωj

Zeq=4+2.5•10-4ωj+10.5-10-7ωj

Zeq=4+2.5•10-4ωj+0.50.25+10-14ω2+10-7ω0.25+10-14ω2j

Zeq=4+0.50.25+10-14ω2+2.5•10-4ω+10-7ω0.25+10-14ω2j

- Convert from complex to polar notation.

Zeq=ZRe+ZImj

ZRe=4+0.50.25+10-14ω2

ZIm=2.5•10-4ω+10-7ω0.25+10-14ω2

|Zeq|=ZRe2+ZIm2

θ=tan-1 ZImZRe

Zeq=Zeq ∠ θ

Sources:

- Electrical Technology (2020 Oct). Resistance, Capacitance & Inductance in Series-Parallel – Equation & Formulas.  https://www.electricaltechnology.org/2020/10/resistance-inductance-capacitance-series-parallel-formulas.html

- LibreTexts Physics (2024). 9.6: Electrical Energy and Power.  https://phys.libretexts.org/Bookshelves/University_Physics/University_Physics_(OpenStax)/University_Physics_II_-_Thermodynamics_Electricity_and_Magnetism_(OpenStax)/09%3A_Current_and_Resistance/9.06%3A_Electrical_Energy_and_Power

- Tech Web (2024).  Resonant Circuits: Resonant Frequency and Q Factor.  https://techweb.rohm.com/product/power-device/si/18332/